| DESCRIPTION - Uniform hollow-cone wide angle spray pattern. - Atomisation becomes finer at higher pressure and wider spray angles. - Swirl chamber designed with counterbore for improved spray quality. - Available in Nylon/PP only - Available in 1⁄4” and 3⁄4” Male |  |  |
| --- | --- | --- |
| SPECIFICATIONS |  |  |
| Thread; eq. orifice [mm]; Flow Rate (lpm) @ Pressure (Bar); Spray Angle (°) @ 3 pressure (bar). TYPE; 1/4"; 3/4”; Outlet; Inlet; 1; 2; 3; 4; 5; 6; 7. 2; -; 1,6; 1,6; 0,45; 0,64; 0,79; 0,91; 1,0; 1,1; 1,2; 115. 4; -; 2,5; 2,1; 0,92; 1,3; 1,6; 1,8; 2,0; 2,2; 2,4; 115. 5; -; 2,5; 2,6; 1,1; 1,6; 2,0; 2,3; 2,5; 2,8; 3,0; 110. 6; -; 2,5; 3,1; 1,3; 1,9; 2,4; 2,7; 3,1; 3,4; 3,6; 110. 10; -; 3,5; 3,5; 2,3; 3,2; 3,9; 4,6; 5,1; 5,6; 6,0; 110. 15; -; 4,0; 3,9; 3,4; 4,8; 5,9; 6,8; 7,6; 8,4; 9,0; 110. 20; -; 5,6; 3,9; 4,5; 6,4; 7,9; 9,1; 10,2; 11,2; 12,1; 110. 25; -; 8,3; 3,8; 4,56; 6,45; 7,9; 9,12; 10,2; 11,2; 12,1; 140. 30; -; 9,3; 4,9; 6,84; 9,67; 11,8; 13,7; 15,3; 16,7; 18,1; 140. 40; -; 9,3; 6,2; 9,12; 12,9; 15,8; 18,2; 20,4; 22,4; 24,1; 140. 60; -; 10,3; 7,9; 13,7; 19,3; 23,7; 27,4; 30,6; 33,4; 36,2; 140. 80; -; 15,9; 7,1; 18,2; 25,8; 31,5; 36,5; 40,8; 44,6; 48,3; 140. 100; -; 15,9; 8,2; 22,8; 32,2; 39,5; 45,6; 51,0; 55,8; 60,3; 140. 120; -; 17,4; 8,6; 27,3; 38,7; 47,4; 54,7; 61,2; 67,0; 72,4; 140. 160; -; 20,6; 9,6; 36,5; 51,6; 63,2; 73,0; 81,6; 89,4; 96,5; 140. 200; -; 20,6; 10,7; 45,6; 64,5; 79,0; 91,2; 102; 112; 121; 140. 240; -; 20,6; 12,1; 54,7; 77,4; 94,7; 109; 122; 134; 145; 140. |
| DIMENSIONS |  |  |
|  | Dimensions. Thread; A; B(Dia); C. (mm). 1/4"; 33.3; 19.1; 24.6. 3/4"; 69.1; 40.0; 47.2. |
| HOW TO ORDER TEXTRA 1/4" WRW Type Material TEXTRA 1/4" WRW 60 Nylon |  |  |